FALL 2024
MEC325: INTRODUCTION TO ENGINEERING DESIGN
DESIGN PROJECT
SITUATIONAL KNOWLEDGE BASE

# Assistive Transport of Children

# when Walking

TEAM 1305

# Team Declaration

We, the undersigned members of Team 1305 in MEC325, agree that:

- All team members have abided by all Ryerson Policies and course rules, and

- One of our members has shared a properly completed Workload Distribution Form with our Teaching Assistant and our instructor.

We furthermore accept that any violation of Ryerson Policy or course rules will lead to a grade penalty or charges of academic misconduct.

| STUDENT NAME | STUDENT NUMBER |
| --- | --- |
| Osama Noureddin | 46598 |
| Andrej Draskovic | 32844 |
| Osman Asif | 35477 |
| Khiem Nguyen | 13735 |
| Yee Yin Kwok | 51256 |
| Codin Nguyen | 37911 |

# Table of Contents

- Research

  - GECUS

  - Stroller Safety Systems, pg. 1

  - Stroller Ergonomics

  - Desired Traits in Strollers

  - Stroller wheels

  - Build Materials

- Images

- References

- Appendix

## Goal:

The goal is to design a human-powered vehicle for newborn to toddler-aged children that enhances the usability, safety, and comfort of traditional strollers while accommodating a broad range of caregivers. This vehicle must be easy to operate by a single untrained user, addressing real-world challenges like public transit, hikes, and public spaces. It should be durable, affordable, and adaptable to the diverse needs of families, including multiple users such as siblings or grandparents. Prioritizing low environmental impact, safety, and ethical considerations throughout its life cycle, the design must also improve the comfort and dignity of both users and co-users.

## Environment:

Strollers are used in various environments, each presenting specific challenges related to terrain, weather, and lighting. In urban settings, sidewalks and streets often involve uneven surfaces and crowded spaces, requiring strollers to be durable and maneuverable. Weather conditions such as rain, snow, and extreme heat demand strollers with weather-resistant materials and sun protection features. Urban areas typically have ample street lighting, but reflectors or built-in lights are helpful for added visibility during low-light conditions (Smith, 2018). In suburban areas, smoother paved pathways and parks are common. Strollers in these environments need to be lightweight for easy transport, and weather conditions like rain or extreme temperatures call for features like rain covers or ventilation (Johnson & Lee, 2020).

Public transportation requires strollers to be compact for easy folding and storage, with stability features to ensure safety in moving vehicles. Weather conditions near bus stops or train stations may involve exposure to rain or wind, necessitating durable design elements (Nguyen, 2022). In shopping centers and stores, maneuverability is essential in tight indoor spaces. These environments are well-lit, but additional storage space for bags and personal items is important (Adams, 2019). Rural and off-road environments introduce challenges like dirt roads, trails, and rough terrains, making larger wheels and better suspension systems necessary. Exposure to rain, wind, and sun requires advanced weather protection (Wilson, 2021).

At beaches, wide wheels or specialized tires are needed to navigate sand. Wind and sun demand sun canopy with UV protection and daylight hours on the beach can be long, requiring strollers that provide adequate shade (Harris, 2020). Airports and travel environments prioritize lightweight and compact designs for easy folding. While airports are typically climate-controlled, outdoor transitions expose users to weather conditions like rain or snow, which necessitates weather protection (Thompson, 2018). Finally, in homes, smooth maneuverability on different floor types (carpet, hardwood) is key, and strollers should be compact for easy storage (Martinez, 2019).

## Competition:

We reviewed a range of existing interventions that can help us understand how they work and the needs of their users in various settings. In most designs, safety and comfort have been enhanced by features such as waterproof material, adjustable harnesses, and strong frame structures. Waterproofing offers protection from rain or moisture, while adjustable harnesses accommodate children's growth and provide secure positioning (Li et al.,2017) Strong structure frames, usually made from lightweight yet robust materials such as aluminum, are important for durability and stability, especially on varied terrains (Wang et al., 2021). These features collectively contribute to handling diverse conditions while ensuring the child's safety and comfort. All designs were made with either 3 or 4 wheels for stability, with 4 wheels being more common. 
	Consider the UPPAbaby Vista V2  [1] for example. It is one of those very versatile and flexible strollers in which you can set up anything to suit the needs of your growing children for a single child or more than one. Because of this, it will be perfect for those parents whose families are expanding. It's also built tough, with a solid frame and big wheels that let it roll easily over rough sidewalks or grassy parks. This stroller is designed for regular use in urban and suburban settings where the expectation is a mix of surfaces. While the more complicated structure of the stroller, such as the various attachments, lifts the price and makes it a bit more difficult to maintain, the additional features make it a reliable choice for those who need something versatile.

Of course, not all strollers have to be this complex. For instance, Jogger City Mini GT2 [2] is more about simplicity and ease of use. It is lightweight and can be folded with one hand, ideal for city parents who need to fold up and store the stroller quickly in tight spaces online buses, or cafes. Compared to the Vista V2, this stroller's simplicity is a plus: its ease of making, cheaper production, and simple maintenance; yet readily durable and easy to use.

Contrarily, the BOB Gear Revolution Flex 3.0 [3] is more specialized for rougher hiking trails or rural areas features like all-terrain wheels and an advanced suspension system. The structure of this stroller would therefore suggest that its target is an environment that is far more rugged and outdoorsy, one in which comfort and safety on uneven or unpredictable terrain are of primary concern. The complexity emanates from the fact that high-quality materials and complicated suspension systems are required, which further add to manufacturing costs and maintenance efforts. However, enhanced durability and specialized design are a necessity for the target users.

Another great product is the Doona Car Seat & Stroller [4] because of its functionality that easily and seamlessly changes between a car seat but also into a stroller. It would be the perfect fit for families always on the go moving from one car to another or other public areas, like an airport or shopping center. Because it is smaller and compact, it would be easy to maneuver around such fast-paced environments. While putting car seat and stroller features into one integrates complexities, the engineering should be precise in satisfying both functions when it comes to safety standards. While this might mean more challenges on the manufacturing and maintenance side, it surely provides ultimate convenience to those traveling quite frequently.

On the other hand, with its high-quality materials, ventilated fabrics, and UV protection, the Silver Cross Wave [5] can offer premium products. This stroller would be suitable for places where luxury and comfort were a priority, such as outdoor parks or beaches. Accordingly, it would imply that it is designed for minimal use in rough environments and boasts a stronger focus on comfort between the toddler and the user. From the materials it is made of to the progressive safety features, it makes the stroller more complex and maintenance-intensive, while it provides a premium service for its users.

Last but not least, the ultra-compact fold mechanism of the GB Pockit+ All-City stroller [6] makes it very suitable for traveling and fast-paced urban life. Its lightweight construction is ideal for indoor usage in tight spaces, such as in subways or shopping centers where maneuverability and portability overnight are needed. Because it only has a sheer construction, it reduces manufacturing complexity to the minimum, making it highly reasonable and easy to service. It can still function effectively in its purpose for the highly valued compact and portable environment.

In conclusion, while many features are shared between these strollers, including weather protection and stout construction, each has features that make them more or less adapted for certain environments. Designs range in complexity from highly specialized functions, like those in the BOB Gear Revolution Flex 3.0, to high-end features found in the Silver Cross Wave. These complexities directly affect manufacturing costs and maintenance requirements, key considerations for our project. Our design will inherit the positive features of these strollers in safety, durability, and adjustability and overcome the shortcomings concerning sustainability and usability by the widest range of caregivers possible. We would like to provide a solution that would still be versatile on various terrains and practical for everyday city use.

## Users:

### User Characteristics

- Demographics:

  - Ages: Primary users are typically parents or caregivers aged 20-45, while secondary users (children) range from newborns to toddlers (0-4 years) (American Academy of Pediatrics, 2020).

  - Genders: Users can be of any gender, though traditionally, mothers are the primary users, and fathers or other caregivers may also participate (Davis & Smith, 2019).

  - Income: Users often have varying income levels, impacting their choice of stroller based on affordability, with budget options starting around $50 and luxury models exceeding $1,000 (Jones, 2021).

  - Backgrounds: Users come from diverse cultural and socio-economic backgrounds, affecting preferences for features like design, functionality, and aesthetics (Li & Liu, 2022).

- Human Factors:

  - Key human factors affecting use include physical strength, dexterity, and height, influencing how easily caregivers can maneuver, fold, or lift the stroller (Smith & Lee, 2021).

  - Cognitive factors such as the ability to multitask (e.g., attending to the child while navigating obstacles) are also significant (Harrison, 2020).

- Abilities and Disabilities:

  - Users may have varying physical abilities, including those with disabilities that impact their ability to lift or push a stroller. Features like adjustable handles or lightweight materials can enhance usability (Martinez, 2019).

  - Considerations for caregivers with mobility impairments are crucial to ensure accessibility (White et al., 2023).

- Emotional and Psychological State:

  - Users might be in varied emotional states, such as excitement when taking their child out or stress when managing multiple tasks. Understanding this can help inform design features like comfort and ease of use (Brown, 2022).

- Co-Users:

  - Children: The child being transported is a primary co-user. Their comfort, safety, and entertainment features (like toys or canopies) can significantly affect the experience (Williams, 2021).

  - Other caregivers: Siblings or grandparents may also use the stroller, necessitating adaptable features for different user sizes and strengths (Davis & Smith, 2019).

- Other Agents:

  - Maintenance personnel: Individuals responsible for cleaning or repairing the stroller may benefit from designs that are easy to disassemble or maintain (Jones, 2021).

  - Retailers: Stores that sell strollers can also be affected by design, as user-friendly products are more likely to result in sales and customer satisfaction (Li & Liu, 2022).

- Adverse Effects:

  - Potential adverse effects on other pedestrians, especially those with disabilities (e.g., the visually impaired), need consideration. Ensuring that strollers are designed to minimize noise and enhance visibility can help avoid accidents (Thompson, 2018).

  - Careful attention to stroller width and turning radius in crowded public spaces can prevent collisions with other users (American Academy of Pediatrics, 2020).

## Strategy:

Market segment

The product targets a broad demographic mentioned in Users, which includes parents, and other members from ages 20-45. The emphasis is to accommodate multiple users of varying physical abilities and income who are searching for a safe, durable, and environmentally friendly solution to assist in the caretaking of newborns and toddlers. Furthermore, the product is marketed to be versatile to different types of Environments and can provide a comfortable use in them. Furthermore, settings such as urban, rural, and suburban areas are where the majority of the target users are located, providing ease of use in areas of public transportation, for strolls around the neighborhood, or at the beach where specific tires are needed to maneuver through the sand with ease. The product must accommodate these settings to provide a versatile option for a wide range of users.

Degree of innovation

This design aims to reconceptualize the stroller to offer a significant improvement in versatility and the ability to adapt to different Environments. Through the integration of many components that seamlessly allow for improved usability for the Users. Furthermore, the product will expand the market by integrating features found on different types of strollers to create a universal product, while also promoting environmentally sustainable practices in the production and life-cycle of the product. The product focuses on not only the approval of the secondary users but also the primary users to provide a comfortable experience for everyone.

Furthermore, the design will integrate advanced ergonomics such as flat seating surfaces which are essential for the child's comfort and to develop posture stability, with various reclining angles to accommodate the child depending on the age. Moreover, features such as footrests and adjustable seating allow for a more safe and universal experience that can accommodate a growing child without the need to purchase a new product (SKB 2.1, 4.6, Users). Ergonomics for the primary users are to be integrated as well, considering aspects such as adjustable handlebars that allow for various users of different heights to comfortably push the strollers in an ergonomic fashion (SKB 2.3, 4.6, Users). As well as ease of storage accessibility to be able to adapt to various Environments where storage space will be necessary for both essential items of the secondary users as well as the primary users (SKB 2.3, Environment).

The product will expand the market by introducing features such as enhanced maneuverability, through the combined integration of swivel and fixed wheels that allow for benefits in both tight spaces and stability in straight-line movement (SKB 5.4), as well as an advanced suspension system that provides a comfortable ride in different Environments where rough terrain can be easily traversed, and can minimize any shocks or vibrations through the implementation of an advanced suspension system (SKB 2.2, 4.1, Environment). Moreover, integrating an expendable design enables caregivers the ability to reduce the space required to store and use the stroller, whereas, in certain environments, compact folding is a huge benefit (SKB 4.2, 4.3, Environment). Furthermore, implementing a design that allows for the ability to switch between different seating settings, such as a cradle, to a modular seat allows for a more universal user experience that can negate the purchase of another stroller as the child matures (SKB 2.1, 4.3, Users).

Lastly, the implementation of environmentally sustainable materials to produce a design that promotes a healthy product lifestyle is essential. Materials such as lightweight aluminum for the frame and UV-resistant polyester for the canopy reduce the environmental impact while also providing a more comfortable experience through a lightweight frame that promotes durability and longevity (SKB 6.1. 6.2, Environment).

Time to market

Given the complex task of implementing many features to produce a universal product, the design is required to have thorough testing in the various Environments such that it proves to be universally adaptable throughout them. Furthermore, to be able to successfully market the stroller, it will take approximately 3-4 years to fully develop the most suitable product that fully integrates all the features seamlessly and effectively.

Year 1-2 will focus on research and development, focusing on key aspects such as ergonomics and prototypes of key features such as the switching between seat settings and the advanced suspension system. Furthermore, research surveys can be distributed to gain insight into what types of products users are looking for. By the end of the 2nd year, initial product versions can be sent to select users for feedback, to gain insight into the usage of the product's features. This early testing can help determine what features are to be tweaked and updated and what needs to be completely redesigned.

Year 3-4 will focus on using all the user data collected to initiate a product launch while also testing the safety conditions as per ASTM International Stroller Safety Standards. Furthermore, additional testing on different terrain Environments is used to determine the validity of suspension, wheels, and the final features. Throughout the development process innovations in features will be documented to add to the current design so that any advancements in technology can be accounted for and implemented properly in the design.

Production

Though it can be a challenge to find the exact number of baby strollers to make a good year relative to competitors, a rough estimate can be found by looking at successful companies in the field. An example successful competitor is the Canadian-based company Dorel Industries, which reported US$699.1 million in revenue for the first six months of 2024 (Dorel News, 2024). The most common price range for their strollers is between $150 and $500, with an overall average cost of around $300. Assuming 15% of the revenue is generated from strollers alone since Dorel also produces car seats and other baby products, it is estimated that 350,000 strollers were produced in six months, and 700,000 for the year. This figure would be a solid indicator of a highly successful year.

Customization

Customization is a selling point for the product, with its vast variety of users, it can be accommodated for different Environments, catering to different preferences. Furthermore, the modular components allow a vast customizable experience that can allow for different seat types to be implemented without adding on or purchasing any additional accessories (Degree of innovation). Moreover, the wheels allow for the ease of maneuverability in many Environments, through rough terrain and long walks/jogs. The stroller containing adjustable handlebars and height adjustment allows for further customization and comfortability for both the child and the parent. Customization ensures that the product caters to the diverse demographic of Users' needs across different activities, allowing for flexibility in seating, comfortability, and maneuverability, allowing for a long-term versatile product.

# Ergonomic

## 2.1 Child Comfort

### Summary of sources:

Child comfort is a necessity for any stroller, without the child feeling comfort, the design of the stroller is insufficient and essentially useless. A child must have the necessary lumbar support as well as the proper pelvic position to promote and develop posture & balance and negate back pain [1]. A flat seating surface accommodates these factors, allowing for an upright position with good posture, and a relaxed position when the child is tired. Furthermore, for newborns, the recommended recline angle is a full 180 degrees, while for older babies it should at least recline to 120-135 degrees. [2] The seating orientation is essential for comfort and health, moreover, a common style of seating, the hammock is an example of poor pelvic position and raises concerns about posture, support, and positioning [1].

Moreover, the child's feet should also require necessary support, a footrest allows for feet to be planted, and comforted rather than dangling, which promotes good posture and reduces any safety concerns [1]. An adjustable footrest allows for various heights and positions of the footrest that can suit the child’s comfort and needs. [1]

Furthermore, adjustable seating heights allow for varying sizes of children which makes the stroller more universal and adaptive to the child's growth. [2] The adjustable heights provide versatility and comfortability for the parent and child, the parent can use the adjustable heights to allow for comfortable seating as the child grows rather than purchasing a completely new stroller to accommodate the child. Some examples of height-adjustable strollers include, Graco Modes Nest Travel System, and Joie Hazel 3-in-1 Stroller which are marketed to have different heights to suit different children.

Shade and weather protection help create a more comfortable and enjoyable environment for the child, and especially during the hot summers where UV resistant cloth is used,  or during the cold winters, it could act as an insulator and keep the child warm. Aspects such as proper ventilation are considered as well in order to not cause any suffocation and maintain proper air circulation through the stroller.  [2]

### Conclusion:

In summary, strollers should implement the correct seating arrangement for children, a flat seating surface that is essential for posture development and a related, reclined position when the child is tired and needs rest. The angle of reclination is important with a full 180 degrees necessary for newborns and at least 120-135 degrees for older children. The design should implement a means to lock the seat at a given angle. Furthermore, footrests should be implemented in strollers to help with balance and comfortability for the child, moreover, adjustability as a whole should be considered as it allows for a more universal stroller that can still allow for a comfortable ride for the child as it grows, allowing for adjustable height settings. Furthermore, adjustable seats that can change positions should be considered in the design that can fit completely switch from a cradle to other seat positions as the child matures. Shade and weather protection is also a consideration as it can protect the child from harmful UV rays and create a more comfortable and insulated environment when it is cold outside. Implementing a design that integrates shade and weather protection seamlessly with the different seat positions can provide an excellent adaptive stroller.

## 2.2 Stability & Suspension

### Summary of sources:

A stroller with a suspension system provides a much higher level of comfort when going out or on walks compared to a stroller without it, allowing for smooth rides on rough terrain as well. [2] There are various types of suspension systems that can be built into the wheels or the frame of the stroller. There is a front-wheel suspension system that absorbs shock at the front wheels, where it is felt the most, a full suspension system that includes on both the front and rear wheels. Moreover, there are air filled tires that combine the suspension system as well as these tires, similar to bike tires. These tires are generally more durable outside of the occasional air pumping and allow for the smoothest ride out of the 3 systems. Furthermore, an alternative to air filled tires is foam-filled tires that have similar results of shock absorption and comfort without the need to worry about punctures or filling with air. A suspension system reduces strain on the child's body as well, especially newborns who cannot balance themselves properly. [2]

- Conclusion:

In conclusion, strollers that implement suspension systems provide a significantly more comfortable ride for both the child and the parent pushing the stroller on rough terrain where it tends to be a challenge to go through. Furthermore, shock absorption improves the child's safety and comfort throughout the ride and makes the ride suitable for long walks or jogs as well. Implementing a suspension system in the stroller is a consideration for the design as it proves to show many benefits. Moreover, the different types of suspension systems show many benefits, however the two clear winners evidently are the air filled tires with suspension system or the foam filled counterpart, which both have the maximum amount of shock absorption and comfort.

## 2.3 Parent Comfort

### Summary of sources:

The parent's comfort is a necessity, being the individual taking care of the child and pushing the stroller, the comfort of the parent is taken into consideration when designing a stroller. Furthermore, many aspects of the stroller must be comfortable for a parent to use in many situations, long strolls, through tough terrain, at night, for elderly people, or for mothers. A stroller must be able to accommodate various heights in order to comfortably push the stroller to reduce back & shoulder strain, furthermore, the handlebars should promote a neutral wrist position and allow for a firm grip. An adjustable handlebar allows for varying heights to be able to comfortably use the stroller and allow for easy maneuvering, moreover, a wider range of adjustment can help not only with height but also the length the bars extend out allowing for more customization. [2]

A stroller should be easy to maneuver and pick up if needed, especially for the elderly, or individuals with any disabilities, a lightweight stroller allows for a more easy ride for the person pushing the stroller. Moreover, stroller weight depends on a multitude of factors, a stroller that incorporates stroller baskets, and umbrellas or shades is generally heavier. Furthermore, materials play the most important role, choosing a lightweight, sustainable material is essential to provide a sufficient stroller, materials such as aluminum or steel are some examples of good materials to use for the frame. Moreover, a lightweight stroller is much more advantageous for users who take the subway or bus. [3]

Storage is essential for strollers allowing the user to store essential items for their children or for themselves and it allows for a stroller to be more resourceful to help store items on the go for when it is needed. Furthermore, the under stroller storage helps lower the center of gravity which allows for a more stable ride for the child and for the parent and helps reduce the risk of tipping over. [2].

### Conclusion:

In summary, parent comfort must be an essential factor when designing a stroller, a parent is the individual who pushes the stroller and thus, it must be a comfortable design. Furthermore, adjustable handlebars in both height and extension must be considered, as it allows for a more universal stroller experience for parents who have different heights and thus can suit the needs of multiple individuals of varying heights who need to push the stroller. The adjustable bars reduce back strain and promote a neutral wrist position which allows for a more enjoyable ride with their children. Moreover, a lightweight design promotes the easy maneuverability of the stroller, especially for individuals with disabilities or seniors who do not necessarily have the strength to push a heavy stroller. A sustainable and lightweight material such as magnesium, or aluminum is the most suitable choice for the frame of a stroller. Moreover, storage is also an essential factor for a stroller as it allows for convenient storage that can be used for the child's items or the parents. The under-stroller storage also doubles as a means of more stability as it lowers the center of gravity.  The integration of a lightweight stroller that implements storage as well is essential for a successful stroller design.

# 3. Government

## 3.1 International Safety Standards

### 3.1.1 General Requirements

#### Summary of Sources

The design and manufacturing of the stroller must abide by the American National Standards Institute (ASTM) Standard Consumer Safety Performance Specification for Carriages and Strollers (Note: Canada also uses these standards for evaluation). The general requirements are as follows:

- The product must not have any sharp points or edges

  - This is to prevent any laceration hazards to the child/occupant or any of the users

- No small parts can be used

  - Preventing choking hazards to the occupant/user. These parts must not have the ability to be removed

  - Small parts must comply with Standard 16 CFR 1501 or image 3.1.1.a

- No lead-based paint can be used

  - This includes all restrictions observed by Standard 16 CFR 1303

- All exposed wood must be smooth and splinterless

- Hinges

  - Hinges must prevent operation when in normal use

  - Considered as part of the frame, must meet the same stress requirements

  - Hinged links must have at least 2 separate and distinct actions to fold

- Holes/Openings

  - If material is less than 9.53mm thick, if it is able to admit a 5.33mm object, it must then also be able to admit a 9.53mm object.

  - If hole does not go all the way through, then this does not apply

- Scissoring/Shearing/Pinching

  - When in intended use position, it should be design so it prevents any injury from any movements

    - It can cause injury if a 5.33mm probe can fit in the gap, but a 9,53mm one cannot

  - Same requirements for saddle hinges

  - Same requirements for canopy hinges

    - However, Canopy stretching/locking devices are exempt

    - Canopy rods and stops are exempt if the rod does not pass over or through the rod

  - Actual frame folding strollers must also comply with the above

    - Carriages (babies lying down) are exempt

    - If the seat is removable to allow for folding, it is also exempt

- Exposed Coil Springs

  - If the spring can create a space of minimum 5.33mm, it must be covered

- Labeling

  - Warnings must be permanent

  - Non-paper labels must not break apart into small parts

- Cord/Strap Length

  - Straps cannot exceed an internal perimeter of 376mm

  - Cannot stretch more than 188mm

  - If they are used to restrain an occupant, carry straps, attachment straps to infant carriers are all exempt

- Strap/Trap/Grab bar

  - The actual bar does not contain foam

  - Can be padded with foam, but must be covered

  - The overeating cannot come off and the foam cannot disintegrate

b ) Conclusion

All designs and fabrication must follow these requirements. If not, this product would not be available for sale or distribution in Canada and most of the world. These general requirements are also safety standards. If they are not followed, users are at risk of damage, injury, or even death.

# 4. Desired Traits

## Smooth Ride & Maneuverability

### Summary of Sources:

Parents consistently seek strollers capable of traversing diverse terrains, aiming to provide a comfortable and pleasant experience for both themselves and their children. Important features include all-wheel suspension, shock absorbers, and oversized wheels, which are essential for navigating uneven sidewalks, gravel paths, and light off-road trails. For example, the UPPAbaby Vista V2 is well-known for its ability to glide smoothly over curbs and rough city streets due to its advanced suspension system (Stroller Insider [1]; Little Baby Gear [2]). Similarly, the Baby Jogger City Mini GT2 features durable "forever-air" rubber tires, offering a stable and comfortable ride on both urban and suburban terrains (Baby Advisor [3]). For more rugged terrains, jogging strollers like the Bob Revolution Flex 3.0, with its air-filled tires and sophisticated suspension, provide excellent maneuverability, although they tend to be heavier (Little Baby Gear [2]).

### Conclusion:

In summary, strollers should focus on enhancing ride quality with features like dual-suspension and large, air-filled tires, making them ideal for caregivers in cities with uneven sidewalks or for active families who enjoy jogging or hiking with their strollers.

## Ease of Use & Portability

### Summary of Sources:

Modern parents, especially those in bustling urban environments or who travel frequently, prioritize strollers that are lightweight, easy to fold, and highly portable. The Babyzen YOYO2 is favored for its ultra-compact fold, one-hand folding feature, and weight of less than 14 pounds, making it ideal for travel and city use. Its all-wheel suspension enhances its performance on uneven urban terrains [4,5]. Another highly recommended option is the Zoe Tour+, weighing just 12 pounds with a quick-fold mechanism, making it convenient for frequent public transport users [6]. The UPPAbaby Vista V2 offers greater versatility, folding with both seats attached. Though heavier, it remains practical due to its all-wheel suspension and one-step fold mechanism, accommodating various family needs [7,8].

### Conclusions

Strollers should integrate one-handed folding systems and utilize lightweight materials such as aluminum to achieve a balance between durability and portability. Compact design is particularly important for city residents or families who frequently use public transportation, making ease of storage a vital aspect of stroller design.

## Modular or Expandable Design

### Summary of Sources:

Parents are increasingly seeking strollers that can adapt as their families grow, particularly those with modular designs that allow for transitions between single and double configurations or combinations with various car seats and bassinets. This modularity helps reduce the need for purchasing multiple strollers over time.

For instance, the Mockingbird Single-to-Double Stroller offers exceptional adaptability, featuring over 22 seating configurations and easily converting from a single to a double stroller by adding a second seat. It supports a wide range of car seats, offering flexibility for growing families [9,10]. Similarly, the UPPAbaby Vista V2 is highly versatile, capable of accommodating a second seat, bassinet, or PiggyBack Ride-Along Board for older siblings. This stroller provides excellent maneuverability and comfort due to its all-wheel suspension and flexible configurations, making it a top choice for long-term family planning [10,12]. Additionally, the Chicco Bravo presents an affordable option by allowing the toddler seat to be removed and replaced with a compatible car seat, converting it into a travel system suitable for families with evolving needs [11,12].

### Conclusions

Strollers need to be versatile enough to handle various setups, catering to a growing family. They should support everything from single to double configurations and be compatible with car seats and bassinets. This adaptability is attractive to families considering more children, positioning the stroller as a worthwhile long-term investment.

## Lightweight and Compact Design

### Summary of Sources:

Parents typically look for strollers that are easy to handle, transport, and store. However, models built for durability or off-road use, like the Bob Revolution Flex 3.0, are often criticized for being too heavy and cumbersome. Strollers that are challenging to lift or fold can pose a major inconvenience, especially for parents who rely on public transportation or live in urban areas with limited space. Even versatile options, such as the UPPAbaby Vista V2, can feel unwieldy when folded due to their size.

Key Issues: Many parents express frustration over the bulkiness and weight of strollers. A stroller that is difficult to carry, particularly when folded, can be a hassle when getting in and out of vehicles, navigating tight spaces, or using public transit.

The Zoe Tour+ addresses this issue by being an ultra-lightweight stroller that weighs approximately 12 lbs and folds up compactly. Nevertheless, strollers like the Bob Revolution continue to be favored for their durability, indicating that a balance between sturdiness and ease of transport is essential.

### Conclusions

Stroller designs should utilize lighter materials, such as advanced aluminum or carbon fiber, to decrease weight while maintaining durability. Additionally, compact folding mechanisms should be improved to minimize the space strollers occupy when folded, making them easier to store in car trunks or small areas. Parents also prefer strollers that can be collapsed with one hand for added convenience while managing their child.

## Affordability and Value

### Summary of Sources:

High-end strollers like the Nuna Demi™ Next and UPPAbaby Vista V2 offer exceptional features and robust construction; however, their price tags, which can range from $1,000 to $1,750, pose a significant challenge for many parents. This concern is heightened by the fact that certain features, such as bassinets or additional seats, may only be utilized for a limited duration.

Parents frequently face the dilemma of balancing cost with essential features, including safety, comfort, and the ability to expand. The elevated prices of premium strollers can render them inaccessible for numerous families, particularly those seeking more economical options that still promise durability.

The Mockingbird Single-to-Double Stroller serves as a prime example of an affordable and adaptable solution. Parents can begin with the basic model and incorporate additional elements, such as a second seat, as their family grows. This modular approach reduces the necessity of acquiring multiple strollers over time, ultimately resulting in cost savings.

### Conclusions

Stroller brands should explore the idea of modular, upgradeable systems that allow parents to buy an affordable base model and add features later, such as bassinets, car seat adapters, or additional seats. This strategy enables parents to manage costs over time instead of facing a large initial expense. To achieve a balance between affordability and functionality, manufacturers should prioritize essential features like safety and portability while providing premium upgrades for those seeking more options.

## Adjustable Features and Comfort

  - Summary of Sources:

One common challenge associated with strollers is their restricted adjustability, which affects the comfort levels of both the child and the caregiver. For example, certain models, such as the Joovy Zoom 360 Ultralight, come with fixed handlebars that can be uncomfortable for caregivers of different heights. Furthermore, strollers that lack adequate seat recline options or footrest adjustments may not be suitable for a growing child, leading to discomfort during longer outings.

A considerable number of strollers do not provide the necessary adaptability for caregivers of various heights, resulting in discomfort and dissatisfaction. Additionally, as children experience rapid growth, the lack of adjustable seats, footrests, and handlebars can make a stroller impractical long before its anticipated lifespan is reached.

Some strollers, like the Baby Jogger City Mini GT2, offer adjustable handlebars, allowing caregivers to customize the stroller's height for more comfortable pushing. However, many other models still lack this essential feature, which restricts their usability for families with caregivers of differing heights.

  - Conclusions

Adjustable handlebars should be considered a fundamental feature in modern strollers, allowing caregivers of different heights to maneuver them effortlessly. Furthermore, strollers ought to offer multiple reclining positions and adjustable footrests to ensure children's comfort as they grow. This level of adaptability not only enhances comfort but also extends the stroller's lifespan, making it a more valuable investment for parents.

# 5. Stroller wheels

## 5.1 Types of Stroller Wheels

- Summary of Sources:

The most common types of stroller wheels are EVA, pneumatic, and air chamber wheels, each with distinct characteristics. EVA wheels consist of an outer plastic layer, making them lightweight. Since the tire portion is entirely composed of EVA, these wheels are resistant to flats; however, they wear down quickly on rough terrain, making them best suited for urban and city environments. Pneumatic wheels feature an outer tire and an inner tube, similar to bike tires. Pneumatic wheels provide extra suspension and more durability but require regular maintenance to maintain air pressure and are prone to flat wheels. Pneumatic wheels are ideal for off-road use due to their superior suspension and durability. Air camber wheels consist of a thick rubber wheel with a large air chamber that offers additional suspension and makes them puncture-proof and maintenance-free. Air chamber wheels are versatile, being suitable for both city and off-road use. [Reference 1]

- Conclusions

In conclusion, air chamber wheels stand out as the best option for stroller design due to their combination of durability, versatility, and low maintenance. Unlike EVA and pneumatic wheels, air chamber wheels offer superior suspension without the risk of punctures or the need for regular maintenance. Their ability to perform well on both city streets and off-road terrain makes them ideal for families seeking a stroller that can handle diverse environments. The puncture-proof design ensures reliability, while the maintenance-free feature provides long-term convenience, making air chamber wheels the most practical and dependable choice for modern strollers.

## 5.2 Stroller wheel configuration

- Summary of Sources:

3-wheel strollers offer excellent maneuverability, particularly on uneven terrain or in tight spaces, thanks to its single front wheel. The all-terrain capability makes it ideal for jogging and outdoor activities, and it performs well on grass, gravel, or rough surfaces. However, it may feel less stable on flat surfaces, especially during sharp turns or sudden stops, and it tends to be bulkier and harder to fold and transport. A 3-wheel design can be less ideal for boarding public transportation because users typically lift the stroller by tipping in onto the rear wheels, followed by the front wheel. With a 3-wheel stroller, this process requires extra effort to maintain lateral balance when tipping on the front wheel, increasing the risk of instability during boarding. In contrast, a 4-wheel stroller provides greater stability due to the even distribution of weight across four wheels, making it ideal for smooth flat surfaces. It is typically more compact and lightweight, making it easier to fold and transport, particularly for city use. [Reference 2]

- Conclusions

Ultimately, a 4-wheel configuration is a superior choice for stroller design due to its enhanced stability, compactness, and ease of use in everyday situations. The even distribution of weight across four wheels provides greater balance and security, especially on flat surfaces, making it ideal for city environments. Additionally, 4-wheel strollers are typically lighter and more compact, making them easier to fold, transport, and store. The extra wheel also improves maneuverability and stability when navigating public transportation, offering a more reliable and practical solution for parents seeking a versatile, easy-to-use stroller.

## 5.3 Stroller wheels size

- Summary of Sources:

When selecting stroller wheel sizes, it is important to match the size to specific needs. Larger wheels (10 to 12 inches) are ideal for rough terrain and jogging strollers, offering better stability and smoother rides, whereas smaller wheels (5 to 8 inches) are more suited for lightweight strollers and urban environments, providing greater maneuverability in tight spaces. Safety standards from the U.S. Consumer Product Safety Commission (CPSC) emphasizes the importance of well-assembled wheels to prevent tip-overs and accidents (Consumer Product Safety Commission [CPSC], n.d; HealthyChildren.org, n.d.). Ergonomic design principles also consider wheel size to ensure smooth mobility and safety (Frontier, n.d.). The best wheel size depends on its intended use: larger wheels for uneven terrain, and smaller wheels for city portability. [Reference 3]

- Conclusions

Ultimately, selecting the right stroller wheel size depends on the intended use and environment. Larger wheels provide superior stability and smoother rides on rough terrain, making them ideal for jogging strollers and all-terrain models, while smaller wheels offer better maneuverability for urban settings and lightweight strollers. Adhering to safety standards from the U.S. Consumer Product Safety Commission (CPSC) ensures that the wheels are robust and well-assembled to prevent accidents.

## 5.4 Fixed vs Swivel Wheel

- Summary of Sources:

Fixed wheels offer superior stability and strength, making them ideal for straight-line movement and heavy loads. They are often simpler in design with fewer moving parts, which reduces the risk of mechanical failure. However, the downside is their limited maneuverability, as they cannot swivel, making it difficult to navigate corners or tight spaces. Turning with fixed wheels requires lifting or pivoting the object, which can be challenging in dynamic environments. [Reference 4]

In contrast, swivel wheels provide excellent maneuverability with their 360-degree rotation, allowing for easy movement in any direction. They are more flexible and convenient for environments requiring frequent turns or navigation around obstacles, such as warehouses or medical settings. Despite their ease of use, swivel wheels are less stable than fixed wheels, especially at higher speeds, and their more complex mechanics may lead to faster wear and the need for more maintenance. A combination of both wheel types is often used to achieve a balance of stability and maneuverability, with fixed wheels providing straight-line control and swivel wheels enabling easier turns. [Reference 5]

- Conclusions

A combination of both swivel and fixed wheels is ideal for stroller design, as it offers the best of both stability and maneuverability. Fixed wheels provide straight-line control and stability, especially on rough or uneven terrain, making the stroller easier to push on longer walks or when carrying heavier loads. Swivel wheels, on the other hand, enhance maneuverability, allowing the stroller to navigate tight spaces, corners, or crowded areas with ease. This balanced configuration ensures that the stroller is versatile enough for a variety of environments, offering parents a smooth, controlled ride without sacrificing convenience or ease of movement.

## 5.5 Wheel Caster Angle

- Summary of Sources:

Caster is the inclination angle measure between the vertical axis and the pivot axis or line of action. It can also be described as “the forward or backward tilt of a line drawn through the top and bottom pivot points during steering” (Shivansh Sabhadiya). As described, the caster can be either forward or backward, which is known as a positive or negative caster. Positive caster is when the point at which the steering axis

intersects the ground is ahead of the contact point of the wheel, whereas negative caster is when it falls behind the contact point. In strollers, a positive caster is typically preferred as it enhances stability, making it easier to maintain a straight line during forward motion. This helps prevent unwanted swaying or veering off course, especially when the stroller is being pushed at higher speeds. A slight positive caster also aids in smoother maneuverability by allowing the wheels to return to a neutral, forward-facing position after making turns, improving the overall handling experience for the user.

However, an excessive positive caster can make the stroller harder to steer, requiring more effort to turn. Therefore, designers need to balance the caster angle to achieve a balance between stability and ease of handling. Negative caster would not be ideal to use in a stroller as it would result in less stable and predictable steering behavior, reducing the usability for a large number of users especially on uneven terrain.

- Conclusions

In the context of baby strollers, the optimal caster angle is carefully chosen based on the specific needs of the user and the terrain the stroller is expected to navigate. For example, strollers designed for urban environments may have a different caster setting compared to those meant for jogging or all-terrain use, where higher stability at varying speeds is essential.

# 6. Build materials

## 6.1 Stroller Frame

- Summary of sources

When deciding what material the main frame of the stroller should be made out of, many things should be considered, such as weight, strength/durability, and cost. Both steel and aluminum are often used for many similar applications as both are quite common materials and offer relatively high tensile strength. Steel tends to be cheaper than many aluminum alloys allowing for decreased production costs, however, many steel alloys tend to weigh 2-3 times the weight of aluminum. Steel also is prone to oxidation when exposed to the elements decreasing its strength and durability, though stainless steel can be used, cost may increase. Aluminum tends to cost more than steel, however, is much lighter whilst maintaining a relatively high tensile strength. Aluminum alloys tend to be resistant to oxidation therefore increasing its longevity. A combination of the two materials would, in theory, allow for the best of both worlds, however, if the two materials are in contact with each other, galling can occur fusing the two parts, and an electron transfer between the two metals can occur such that the aluminum would weakening, therefore when looking for fasteners, the material that they are made up of has to be considered. [Reference 1]

- Conclusion

Ultimately, a full aluminum frame would offer the best cost-performance for the product without having a great compromise on durability. Aluminum would offer a lightweight design allowing for a better user experience, and the inherent oxidation resistance would provide a long-lasting product for the consumer.

## 6.2 Stroller carriage

a. Summary of sources

The material of the stroller carriage is quite important for the comfort of the passenger and the protection of the user from the elements. Natural materials such as cotton or wool would offer decent durability and comfort as the softer fibers would be better for sensitive skin. Cotton and wool do not do well in the elements as they are quite water absorbent, this will also lead to the absorption of any fluids produced by the user causing increased maintenance and increased chance for bacterial growth within the material. Cotton and wool are quite susceptible to mildew resulting in the material deteriorating over time if mildew were to occur. Synthetic materials such as polyester also offer decent durability and tend to be water resistant allowing for better protection against the elements. The inherent water resistance of the material allows for an easier clean-up if any accidents were to occur, also decreasing the chance of bacterial growth. However, due to the stiff fibers and chemicals used to treat the fibers, polyester fabrics may irritate users with more sensitive skin resulting in rashes. [Reference 2]

### b. Conclusion

A combination of the two fabrics would provide the best of both worlds allowing for good protection against the elements. By having the outer layer be multiple thick layers of polyester fabric, it allows for a more protective outer layer utilizing the water resistance of the fabric to help protect the passenger from the elements. Having a thicker layer of fabric as an outer layer, would allow for the fabric to protect against UV rays, as the inherent UPF 30+ rating of the fabric would block the UV rays. Also having an interior layer of cotton or wool would help increase the comfort of the passenger and decrease the chance of skin irritation.

## 6.3 Packaging

- Summary of sources

With the advent of online shopping and deliveries, there has been an influx of disposable packaging filling landfills, as a result when designing a product the packaging material must be taken into consideration. A promising substitute to the current plastic packaging that is most commonly used, is mycelium composite materials. Mycelium packaging shows promising results, as it displays relatively high compressive strength allowing it to replace styrofoam packaging materials. As mycelium packaging is also biodegradable, it can be used as a more environmentally friendly option for packaging. A cardboard box will still be required, however having biodegradable packaging material will help decrease the environmental impact of the packaging. Due to mycelium packaging being relatively new and not widely adapted, an increased production cost will occur as a result of the technology for production being quite limited and development quite small for the product. [Reference 3][Reference 4]

### Conclusion

Using both cardboard and mycelium packaging would allow for biodegradable packaging. With the combination of minimal packaging by grouping loose parts together and by using folded packaging to minimize the use of adhesives, the packaging can then be potentially upcycled, more easily recycled, or composted allowing for a more environmentally friendly product.

# images

Fig. 3.1.1.a: Small Parts cylinder in compliance with 16 CFR 1501

# Appendix A: GECUS

Adams, M. (2019). Design considerations for urban mobility devices: A user-centered approach. Journal of Urban Mobility, 12(3), 45–58.

Harris, T. (2020). Outdoor stroller design: Navigating beach and off-road terrains. Outdoor Equipment Design Review, 8(4), 102–116.

Johnson, R., & Lee, C. (2020). Suburban stroller designs for family-friendly communities. Journal of Suburban Studies, 10(1), 22–34.

Martinez, P. (2019). Indoor stroller use: Considerations for maneuverability and safety. Journal of Home Design, 6(2), 50–66.

Nguyen, T. (2022). Public transportation challenges for stroller users: An ergonomic perspective. Transportation Journal, 14(1), 75–90.

Smith, J. (2018). Urban stroller design: Adapting to environmental challenges in city landscapes. Design & Society, 11(2), 85–98.

Thompson, A. (2018). Airport travel and stroller design: Innovations for mobility and convenience. Journal of Travel Design, 7(3), 60–72.

Wilson, B. (2021). Rural stroller design: Durability and comfort in off-road environments. Journal of Product Design, 15(4), 120–135.

Dorel reports second quarter 2024 results (2024) Dorel Industries. Available at: https://www.dorel.com/blogs/news/dorel-reports-second-quarter-2024-results (Accessed: 11 October 2024).

American Academy of Pediatrics. (2020). Caring for your baby and young child: Birth to age 5 (6th ed.). Bantam.

Brown, R. (2022). The emotional landscape of parenting: Designing products for caregivers. Journal of Parenting Studies, 15(3), 45-58.

Davis, K., & Smith, A. (2019). Gender dynamics in parenting products: An analysis of stroller usage. Journal of Family Studies, 12(4), 65-78.

Harrison, L. (2020). Cognitive load in parenting: Balancing tasks with child care. Journal of Child Development, 9(2), 123-136.

Jones, T. (2021). The economics of stroller ownership: Cost, features, and market trends. Journal of Consumer Economics, 18(1), 22-39.

Li, Q., & Liu, Y. (2022). Cultural influences on parenting practices: A stroller case study. International Journal of Parenting, 7(1), 11-29.

Martinez, P. (2019). Accessibility in baby gear: Design considerations for diverse users. Journal of Inclusive Design, 5(2), 34-47.

Smith, J., & Lee, C. (2021). Ergonomic considerations in child transportation devices. Journal of Human Factors and Ergonomics, 13(4), 78-90.

Thompson, A. (2018). Stroller safety and user awareness in urban settings. Transportation Research Part F, 14(3), 45-55.

White, R., Jackson, M., & Green, S. (2023). Designing for all: Stroller adaptations for caregivers with disabilities. Journal of Assistive Technology, 14(1), 60-73.

Williams, T. (2021). The role of play in stroller design: Keeping children engaged during travel. Journal of Child Psychology, 10(3), 90-99.

Li, Z., Chen, W., and Wang, Y., "Development of Child-Safe Equipment with Waterproof and Adjustable Features," J. Mech. Des., vol. 139, no. 6, pp. 1125–1135, 2017.

Wang, D., Xie, C., Liu, Y., and Xu, W., "Multi-objective Collaborative Optimization for the Lightweight Design of an Electric Bus Body Frame," Automot. Innov., vol. 3, no. 4, pp. 250-259, 2021.

# Appendix B: Ergonomic

[1] “Ergonomic strollers – confusion in definition,” Learn to Move, https://www.learntomove.se/en/project/ergonomiska-barnvagnar-definitionsforvirring/#:~:text=The%20key%20to%20sitting%20upright,from%20the%20front%20or%20back.

[2] Picket&Rail Custom Sofas & Furniture, “Baby Stroller - Top 20 must have features,” Picket&Rail Custom Sofas & Furniture, https://picketandrail.com/blogs/baby-kids-blog/the-top-20-best-baby-stroller-features#:~:text=An%20adjustable%20footrest%20is%20an,baby’s%20%20feet%20are%20%20adequately%20supported.

[3] “Choose the right stroller for your family,” Consumer Reports, https://www.consumerreports.org/babies-kids/strollers/buying-guide/#what-other-factors-should-i-consider

[4] J. Spurrier, “How to pick a stroller that is right for you,” GearLab, https://www.babygearlab.com/topics/getting-around/best-full-size-stroller/buying-advice

[5] Mmldigi, “A comprehensive guide to when can baby sit in Stroller,” River Baby Group, https://www.riverbabygroup.com/a-comprehensive-guide-to-when-can-baby-sit-in-stroller/#:~:text=Babies%20have%20limited%20head%20and,seats%20when%20using%20a%20stroller.&text=Babies%20gain%20better%20head%20control,a%20stroller%20for%20added%20support.

# Appendix C: Government

# [1] “Standard Consumer Safety Performance Specification for carriages and Strollers,” F833, https://www.astm.org/f0833-21.html

[2] “The Federal Register,” Code of Federal Regulations, https://www.ecfr.gov/current/title-16/chapter-II/subchapter-C/part-1501

[3] “The Federal Register,” Federal Register, https://www.ecfr.gov/current/title-16/chapter-II/subchapter-B/part-1303

# [4] K. F. and M. Bradley, “Looking for a safe stroller? assess these 5 features before you buy.,” Consumer Reports, https://www.consumerreports.org/babies-kids/strollers/important-stroller-safety-features-to-look-for-a2549523505/

[5] H. Canada, “Government of Canada,” Canada.ca, https://www.canada.ca/en/health-canada/services/infant-care/strollers-carriages.html

[6] K. M. Cronan, Ed., “Choosing safe baby products: Strollers (for parents) | nemours kidshealth,” KidsHealth, https://kidshealth.org/en/parents/products-strollers.html#:~:text=The%20safest%20design%20is%20a,used%20in%20the%20reclined%20position

# Appendix D: Desired Traits

[1] Stroller Insider, "Best All Terrain Strollers Of 2024: Offer Comfort To Your Baby," available: https://strollerinsider.com/all-terrain-strollers.

[2] Little Baby Gear, "Best All-Terrain Strollers (2024): For Gravel, Dirt Roads," available: https://www.littlebabygear.com/all-terrain-strollers.

[3] Baby Advisor, "The 8 Best All-Terrain Strollers for Adventurous Parents," available: https://www.babyadvisor.org/all-terrain-strollers.

[4] Strolleria, “UPPAbaby Vista V2 vs. Babyzen YOYO2,” Strolleria, Mar. 2023. [Online]. Available: https://strolleria.com.

[5] Stroller Insider, “UPPAbaby VISTA V2 Review [2024],” Stroller Insider, 2024. [Online]. Available: https://strollerinsider.com.

[6] BabyStrollerTrend, “Zoe Tour+ Stroller Review,” BabyStrollerTrend, 2023. [Online]. Available: https://babystrollertrend.com.

[7] UPPAbaby, “Vista V2 Stroller,” UPPAbaby, 2024. [Online]. Available: https://uppababy.com.

[8] Stroller Insider, “UPPAbaby Vista V2 Stroller Review: Everything You Need to Know,” Stroller Insider, 2024. [Online]. Available: https://strollerinsider.com.

[9] Mockingbird. "Best Double Strollers for Infants and Toddlers." Mockingbird, 2024. Available: https://hellomockingbird.com.

[10] Strolleria. "Mockingbird vs. UPPAbaby Vista V2 Stroller Comparison." Strolleria, 2024. Available: https://strolleria.com.

[11] Mommyhood101. "Mockingbird Single-to-Double 2.0 Stroller Review." Mommyhood101, 2024. Available: https://mommyhood101.com.

[12] Parents. "The 6 Best Convertible (Single to Double) Strollers of 2024." Parents, 2024. Available: https://parents.com.

# Appendix E: Stroller Wheels

[1] ABC Design. (n.d.). The right wheels for your pushchair. ABC Design. https://www.abc-design.com/en/news/news/article/the-right-wheels-for-your-pushchair

[2] Baby Stroller Lab. (n.d.). 3-wheel vs 4-wheel stroller: Which one is right for you and your baby? Baby Stroller Lab. https://babystrollerlab.com/3-wheel-vs-4-wheel-stroller/

[3] Our Start. (n.d.). The pros and cons of having a three wheel vs four wheel stroller. Our Start. https://www.ourstart.com/the-pros-and-cons-of-having-a-three-wheel-vs-four-wheel-stroller/

[4] Consumer Product Safety Commission (CPSC). (n.d.). Carriages and strollers business guidance & small entity compliance guide. CPSC.gov. Retrieved October 5, 2024, from https://www.cpsc.gov/Business--Manufacturing/Business-Education/Business-Guidance/Carriages-and-Strollers

[5] HealthyChildren.org. (n.d.). How to choose a safe baby stroller. Retrieved October 5, 2024, from https://www.healthychildren.org

[6] Frontiers. (n.d.). The modularization design and autonomous motion control of a new baby stroller. Frontiers in Psychology. Retrieved October 5, 2024, from https://www.frontiersin.org/articles/10.3389/fpsyg.2023.1030650/full

# Appendix F: Build Materials

[1] Miner, Billy. “Steel vs. Aluminum: Which Metal Is Best for Your Product?” Gabrian, 23 Mar. 2023 https://www.gabrian.com/steel-vs-aluminum/

[2] Militky, J. “The Chemistry, Manufacture and Tensile Behaviour of Polyester Fibers.” ScienceDirect, 27 Mar. 2014 https://www.sciencedirect.com/science/article/abs/pii/B9781845693879500096

[3] Balaeș, Tiberius et al. “Mycelium-Composite Materials-A Promising Alternative to Plastics?.” Journal of fungi (Basel, Switzerland) vol. 9,2 210. 6 Feb. 2023 https://www.ncbi.nlm.nih.gov/pmc/articles/PMC9965147/

[4 ]Pohan, Jennifer Nathania, et al. “Mushroom Mycelium-Based Biodegradable Packaging Material: A Promising Sustainable Solution for Food Industry.” E3S Web of Conferences, EDP Sciences, 15 Sept. 2023 https://www.e3s-conferences.org/articles/e3sconf/pdf/2023/63/e3sconf_icobar23_02128.pdf

Only the last 5 digits of the student number are required.